Discontinuous Galerkin  method in modeling  damage dynamics

Ioan R. Ionescu1, Quriaky Gomez1) and Oana Ciobanu2 (SAFRANE, France)

1LSPM, Paris Nord University, France

2 SAFRAN, France

We develop  here a  discontinuous Galerkin (DG) method  to investigate the wave propagation in heterogenous brittle materials,  modeled by  a  linear elastic body  with a family of (small scale) cracks  in uni-lateral  frictional  contact.  In the classical finite element technique an inner boundary condition requires a geometrical treatment, hence the computational effort became very important for a large number of micro-cracks.  In contrast, in the DG method the inner boundary conditions are modeled by the flux choice without an additional computational cost, even for many micro-cracks.  Due the non-linear conditions (non penetration, friction, etc) on the inner boundaries (micro-cracks) the problem is nonlinear.  That is why we have used a numerical upscaling homogenization technique   to find the effective properties of the damaged material.  We   analyze the wave propagation (speed, amplitude and wavelength) of micro-cracked materials in simple or more complicated configurations.